SIMATS ENGINEERING

ASSESSMENT TOOL 1 - CODING CHALLENGE SOLUTIONS

| Name: | SHARUN.SU |
| --- | --- |
| Register No: | 192571012 |
| Course Code: | 0703 |
| Course Outcome: | CO4 - Design network topologies (star, mesh, bus, hybrid) |

# Question 1: Star Topology Generator

Develop a Python program that generates a Star Topology for a given number of hosts connected to a central switch, displaying host label, switch name, and Cat6 UTP cable length for each host, and verifying connectivity.

## Python Code

import random

def generate_star_topology(num_hosts):

switch_name = "Central-Switch"

print(f"Star Topology with {num_hosts} hosts")

print("=" * 40)

connected_hosts = []

for i in range(1, num_hosts + 1):

host_label = f"Host-{i}"

cable_length = round(random.uniform(1.0, 20.0), 2)

connected_hosts.append(host_label)

print(f"{host_label} --- Cat6 UTP ({cable_length} m) --- {switch_name}")

print("\nASCII Representation:")

print(" " * 10 + switch_name)

for host in connected_hosts:

print(f"   |--- {host}")

print(f"\nVerification: {len(connected_hosts)} of {num_hosts} hosts "

f"are connected through {switch_name}.")

print("\nExplanation:")

print("Every host has its own dedicated Cat6 UTP link to the central switch.")

print("If one cable or host fails, the rest of the network keeps working,")

print("and the switch isolates each link's collision domain, giving every")

print("host full dedicated bandwidth. This makes the star topology reliable")

print("and easy to troubleshoot for a local area network.")

if __name__ == "__main__":

n = int(input("Enter number of hosts: "))

generate_star_topology(n)

## Sample Output

Enter number of hosts: 3

Star Topology with 3 hosts

========================================

Host-1 --- Cat6 UTP (12.45 m) --- Central-Switch

Host-2 --- Cat6 UTP (7.31 m) --- Central-Switch

Host-3 --- Cat6 UTP (18.02 m) --- Central-Switch

ASCII Representation:

Central-Switch

|--- Host-1

|--- Host-2

|--- Host-3

Verification: 3 of 3 hosts are connected through Central-Switch.

## Explanation

In a star topology every host connects to the central switch through its own dedicated Cat6 UTP cable. Because each link is independent, a cable or host failure does not affect other hosts, and the switch keeps each connection in its own collision domain, giving every host full dedicated bandwidth. This isolation and centralised control make the star topology reliable and easy to manage in a local area network.

# Question 2: CSMA/CD Simulation

Develop a Python program to simulate CSMA/CD in a star topology with four hosts, simulating transmission attempts, detecting collisions, calculating random back-off intervals, and logging every event to csma_log.txt.

## Python Code

import random

import time

def csma_cd_simulation(num_hosts=4, rounds=3):

hosts = [f"Host-{i}" for i in range(1, num_hosts + 1)]

log = open("csma_log.txt", "w")

def write(msg):

print(msg)

log.write(msg + "\n")

write("CSMA/CD Simulation - Star Topology (Central Switch)")

write("=" * 55)

for round_num in range(1, rounds + 1):

write(f"\n--- Transmission Round {round_num} ---")

transmitting = random.sample(hosts, k=random.randint(1, num_hosts))

write(f"Hosts attempting to transmit: {transmitting}")

if len(transmitting) > 1:

write("COLLISION DETECTED between: " + ", ".join(transmitting))

backoffs = {}

for host in transmitting:

backoffs[host] = round(random.uniform(0.1, 1.0), 2)

write(f"{host}: back-off interval = {backoffs[host]}s")

time.sleep(0.05)

for host in sorted(transmitting, key=lambda h: backoffs[h]):

write(f"{host} retransmitting after back-off... Success.")

else:

write(f"{transmitting[0]} transmitted frame successfully. No collision.")

write("\nExplanation:")

write("Each host senses the channel before transmitting (Carrier Sense).")

write("If two or more hosts transmit at the same time, a collision is")

write("detected, every involved host stops and waits a random back-off")

write("interval before retrying. This randomised back-off spreads out")

write("retransmissions in time, minimising repeated collisions and")

write("keeping Ethernet traffic efficient.")

log.close()

if __name__ == "__main__":

csma_cd_simulation()

## Sample Log Output (csma_log.txt)

CSMA/CD Simulation - Star Topology (Central Switch)

=======================================================

--- Transmission Round 1 ---

Hosts attempting to transmit: ['Host-2', 'Host-4']

COLLISION DETECTED between: Host-2, Host-4

Host-2: back-off interval = 0.42s

Host-4: back-off interval = 0.77s

Host-2 retransmitting after back-off... Success.

Host-4 retransmitting after back-off... Success.

--- Transmission Round 2 ---

Hosts attempting to transmit: ['Host-1']

Host-1 transmitted frame successfully. No collision.

## Explanation

Each host listens to the shared medium before sending (Carrier Sense). When two or more hosts transmit at the same instant, the switch/medium detects a collision; every involved host halts transmission and waits a randomly chosen back-off interval before retrying. Randomising the back-off time keeps the same hosts from colliding again immediately, which is exactly how CSMA/CD reduces repeated collisions and keeps Ethernet networks efficient even when several hosts share the same segment.

# Question 3: Client-Server MAC Address Table Simulation

Develop a client-server application using Python's socket library to simulate two hosts communicating through a central switch, where the server maintains and displays a simulated MAC address table after every successful transmission.

## Python Code - server.py

import socket

import threading

mac_table = {}

def handle_client(conn, addr, port_id):

while True:

try:

data = conn.recv(1024)

if not data:

break

frame = data.decode()

src_mac, message = frame.split("|", 1)

mac_table[src_mac] = port_id

print(f"Frame received from {src_mac} on Port {port_id}: {message}")

print("Updated MAC Address Table:")

for mac, port in mac_table.items():

print(f"  {mac} -> Port {port}")

conn.sendall(f"ACK:{message}".encode())

except ConnectionResetError:

break

conn.close()

def start_server(host="127.0.0.1", port=5000, max_hosts=2):

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

server.bind((host, port))

server.listen(max_hosts)

print(f"Central Switch listening on {host}:{port}")

port_id = 1

while port_id <= max_hosts:

conn, addr = server.accept()

threading.Thread(

target=handle_client, args=(conn, addr, port_id), daemon=True

).start()

port_id += 1

if __name__ == "__main__":

start_server()

## Python Code - client.py

import socket

def start_client(mac_address, message, host="127.0.0.1", port=5000):

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client.connect((host, port))

frame = f"{mac_address}|{message}"

client.sendall(frame.encode())

response = client.recv(1024).decode()

print(f"Server response: {response}")

client.close()

if __name__ == "__main__":

start_client("AA:BB:CC:00:00:01", "Hello from Host-1")

## Sample Output (server side)

Central Switch listening on 127.0.0.1:5000

Frame received from AA:BB:CC:00:00:01 on Port 1: Hello from Host-1

Updated MAC Address Table:

AA:BB:CC:00:00:01 -> Port 1

Frame received from AA:BB:CC:00:00:02 on Port 2: Hello from Host-2

Updated MAC Address Table:

AA:BB:CC:00:00:01 -> Port 1

AA:BB:CC:00:00:02 -> Port 2

## Explanation

An Ethernet switch performs MAC learning by inspecting the source MAC address of every incoming frame and recording which port it arrived on. In this simulation, the server reads the source MAC from each client's frame and stores the (MAC, port) pair in a table, printing the updated table after every transmission. For forwarding, a real switch checks the destination MAC against this table: if it is known, the frame is forwarded only to that port; if unknown, the frame is flooded to all ports, which is why building an accurate MAC table is essential for efficient frame forwarding.

# Question 4: StarSwitch Class Implementation

Design and implement a Python class named StarSwitch modelling an Ethernet switch, with add_port, remove_port, and broadcast methods, maintaining active ports and logging broadcast operations.

## Python Code

class StarSwitch:

def __init__(self):

self.active_ports = {}

self.broadcast_log = []

def add_port(self, host_id):

self.active_ports[host_id] = True

print(f"Port added: {host_id} connected to switch.")

def remove_port(self, host_id):

if host_id in self.active_ports:

del self.active_ports[host_id]

print(f"Port removed: {host_id} disconnected from switch.")

else:

print(f"{host_id} is not connected.")

def broadcast(self, source_id, frame):

if source_id not in self.active_ports:

print(f"Broadcast failed: {source_id} is not connected.")

return

destinations = [h for h in self.active_ports if h != source_id]

self.broadcast_log.append((source_id, destinations, frame))

print(f"Broadcasting from {source_id}: '{frame}'")

for dest in destinations:

print(f"  -> {dest} received the frame.")

if __name__ == "__main__":

switch = StarSwitch()

switch.add_port("Host-1")

switch.add_port("Host-2")

switch.add_port("Host-3")

switch.broadcast("Host-1", "ARP Request: Who has 192.168.1.5?")

switch.remove_port("Host-2")

switch.broadcast("Host-3", "Data Frame")

## Sample Output

Port added: Host-1 connected to switch.

Port added: Host-2 connected to switch.

Port added: Host-3 connected to switch.

Broadcasting from Host-1: 'ARP Request: Who has 192.168.1.5?'

-> Host-2 received the frame.

-> Host-3 received the frame.

Port removed: Host-2 disconnected from switch.

Broadcasting from Host-3: 'Data Frame'

-> Host-1 received the frame.

## Explanation

The StarSwitch class keeps track of every currently connected host in active_ports, so add_port and remove_port simply update this record. The broadcast method sends a copy of a frame to every connected port except the source. Broadcasting is significant in Ethernet because protocols such as ARP need to reach an unknown destination MAC address; flooding the frame to all ports guarantees it reaches the intended host even before the switch has learned that host's specific port, which is also what a real switch does whenever a destination MAC is not yet present in its address table.