SIMATS ENGINEERING

ASSESSMENT TOOL 1 - ANSWER SCRIPT

| Name | SHARUN.SU |
| --- | --- |
| Register No. | 192571012 |
| Course Code | CSA07 (0703) |
| Course Name | Computer Networks |
| Course Outcome | CO5 - Design network topologies using networking devices, transmission media, and cabling standards |

# Question 1: Rural Branch Office Connectivity (10 Marks)

A multinational company is opening a branch in a remote rural location without wired high-speed internet. It must support cloud applications, video conferencing, and secure communication with headquarters, on a limited budget, with uninterrupted operations.

Recommended Internet Connection

- Primary link: 4G/5G Fixed Wireless Access (FWA) using a business-grade cellular router, since most rural areas today have cellular tower coverage even without fiber.

- Backup link: Satellite broadband (e.g., VSAT or LEO satellite service) as a failover connection for locations with poor cellular signal or during tower outages.

- Deploy both links through an SD-WAN / dual-WAN router with automatic failover so the office is never fully offline.

Recommended Networking Devices

- Dual-SIM/dual-WAN business router with SD-WAN and load-balancing/failover capability.

- Next-generation firewall (NGFW) for perimeter security and VPN termination.

- Layer 2 managed switch for wired office connectivity.

- WiFi 6 access point for reliable indoor wireless coverage.

- UPS with sufficient backup time to keep networking gear running during power interruptions common in rural areas.

Recommended Security Mechanisms

- Site-to-site IPSec VPN tunnel between the branch router/firewall and headquarters for secure data exchange.

- WPA3 encryption on the wireless network and MFA for access to cloud applications.

- NGFW with intrusion prevention (IPS) and content filtering to block malicious traffic over the cellular/satellite link.

Justification

- Bandwidth: Video conferencing typically needs 2-4 Mbps per stream; modern 4G/5G FWA comfortably meets this, and QoS on the router can prioritize video and VPN traffic over less critical traffic.

- Reliability: Dual-WAN failover between cellular and satellite ensures business continuity even if one carrier has an outage, satisfying the uninterrupted-operations requirement.

- Cost: Avoiding new fiber trenching (very expensive in remote areas) and instead using subscription-based cellular/satellite service converts a large CAPEX into a manageable monthly OPEX, fitting the limited budget.

- Scalability: SD-WAN configuration allows the company to add bandwidth, switch carriers, or add more branch sites later without replacing hardware, and cloud-based applications mean minimal additional on-site infrastructure is needed as the branch grows.

# Question 2: Campus-Wide Wireless Network Design (10 Marks)

A university needs campus-wide WiFi for 3,000+ students across classrooms, labs, libraries, and hostels, supporting many simultaneous users, minimizing interference, ensuring secure access, and using only a limited number of access points (APs).

Access Point Placement

- Conduct a wireless site survey to generate a heat map of each building and place APs only where user density is highest (lecture halls, libraries, labs, hostel common areas), avoiding redundant coverage.

- Use ceiling-mounted omnidirectional APs in classrooms and hostel corridors for even coverage, and higher-capacity APs with sectorized/directional antennas in large open areas such as libraries and auditoriums to serve more concurrent users per AP.

- Overlap adjacent AP cells by about 15-20% only, enough for seamless roaming without creating co-channel interference.

- Choose WiFi 6 (802.11ax) access points, since features like OFDMA and MU-MIMO let each AP serve many more simultaneous devices, reducing the total number of APs needed to fit the budget.

Frequency Band Selection

- Operate dual-band APs (2.4 GHz and 5 GHz) with band steering enabled to automatically push capable devices to 5 GHz.

- Prioritize the 5 GHz band in dense areas (libraries, classrooms) since it offers many more non-overlapping channels and higher throughput with less interference.

- Reserve 2.4 GHz mainly for wider-range coverage in hostels and outdoor areas where fewer devices connect at once.

- Use non-overlapping channels (1, 6, 11 on 2.4 GHz) and 20-40 MHz channel widths on 5 GHz, assigned automatically via a wireless LAN controller to minimize channel interference across the campus.

Authentication Methods

- Deploy WPA2/WPA3-Enterprise with 802.1X authentication against a central RADIUS server integrated with the university's student/staff directory (LDAP/Active Directory), so each user authenticates with their own credentials.

- Use separate SSIDs and VLANs for students, staff, and guests, so guest traffic is isolated from academic and administrative systems.

- Provide a time-limited captive portal for visitor/guest access instead of exposing the main secure SSID.

How the Design Balances Coverage, Performance, Security, and Cost

- Coverage: Site-survey-driven placement ensures no dead zones while using the minimum number of APs.

- Performance: WiFi 6 (OFDMA/MU-MIMO) plus 5 GHz prioritization allows each AP to serve hundreds of simultaneous connections without congestion.

- Security: 802.1X/RADIUS authentication with per-user credentials and VLAN segmentation ensures only authorized users access sensitive academic networks.

- Cost: Choosing fewer, higher-capacity WiFi 6 APs managed by a single wireless controller reduces both the number of devices purchased and ongoing management overhead, meeting the budget constraint.

# Question 3: Secure Network Architecture for a Financial Institution (10 Marks)

A financial institution must upgrade its internal network to improve cybersecurity for sensitive customer data and online banking access, while complying with strict regulations and staying within performance and cost constraints.

Network Segmentation

- Segment the network into VLANs: customer database/core banking servers, employee workstations, an online-banking DMZ, and a guest/visitor network.

- Place internet-facing online banking and web servers in a DMZ, isolated from the internal LAN by two layers of firewalling (external and internal firewall).

- Apply micro-segmentation around the most sensitive segment (the customer database tier) so that even if another internal segment is compromised, the database tier remains isolated.

Firewall Deployment

- Deploy a next-generation firewall (NGFW) at the network perimeter for deep packet inspection, application awareness, and threat intelligence-based filtering.

- Deploy an additional internal segmentation firewall between VLANs (a zero-trust approach) so that lateral movement between segments is blocked and inspected, rather than trusting all internal traffic by default.

Authentication Methods

- Enforce multi-factor authentication (MFA) for all employees accessing sensitive customer data or online banking systems.

- Apply role-based access control (RBAC) so employees only access the data and systems required for their job function.

- Use 802.1X port-based network access control for wired/wireless connections and certificate-based authentication for administrative and server access.

Intrusion Detection Mechanisms

- Deploy a network-based Intrusion Detection/Prevention System (IDS/IPS) at the perimeter and DMZ to monitor for known attack signatures and abnormal traffic patterns.

- Feed firewall, IDS, and server logs into a centralized SIEM system for real-time correlation, alerting, and the audit trail required by financial regulations.

- Deploy endpoint detection and response (EDR) on employee workstations and servers as an additional detection layer.

Justification

- Security: Layered defense (segmentation + dual-firewall DMZ + MFA + IDS/IPS + SIEM) protects sensitive customer and banking data from both external attackers and internal lateral movement.

- Performance: Hardware-accelerated NGFWs and applying deep inspection only where it is needed (DMZ and sensitive VLANs) avoid bottlenecking normal employee traffic, while VLAN segmentation reduces broadcast overhead.

- Compliance: RBAC, MFA, and SIEM-based audit logging directly support regulatory requirements such as segregation of duties and traceable access records expected of financial institutions.

- Cost: Reusing existing managed switches for VLAN segmentation, using open-source IDS tools where suitable, and rolling out MFA first to high-privilege accounts keeps the upgrade within budget while still meeting the core security requirements.