SIMATS ENGINEERING

Department of Information Technology

ASSESSMENT TOOL 1 — ANALYTICAL PROBLEM SOLVING

Course Name: Computer Networks     |     Course Code: CSA07

Name: Sharun.SU     |     Register No: 192571012     |     Course Code: 0703

Course Outcome: CO2 — Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology (BL3)

Assessment Tool Weightage: 40%

# Question 1

A university network 192.168.20.0/24 is divided into three lab subnets: Lab A (/26), Lab B (/27), Lab C (/28). Blocks are assigned sequentially starting from the base network address.

Solution

Block sizes: /26 → 256 − 192 = 64 addresses; /27 → 32 addresses; /28 → 16 addresses. Subnets are allotted one after another, largest first, so each new block starts right after the previous block's broadcast address.

| Lab / Mask | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- |
| Lab A /26 | 192.168.20.0 | 192.168.20.1 – .62 | 192.168.20.63 | 62 |
| Lab B /27 | 192.168.20.64 | 192.168.20.65 – .94 | 192.168.20.95 | 30 |
| Lab C /28 | 192.168.20.96 | 192.168.20.97 – .110 | 192.168.20.111 | 14 |

Usable hosts = 2^(host bits) − 2 (network + broadcast reserved): Lab A = 2^6 − 2 = 62; Lab B = 2^5 − 2 = 30; Lab C = 2^4 − 2 = 14.

# Question 2

System 1: 192.168.1.130 / 255.255.255.192 (/26).  System 2: 192.168.1.190 / 255.255.255.192 (/26).

Solution

255.255.255.192 = /26 → block size 64. Subnet boundaries in the last octet: 0, 64, 128, 192.

| System | Given IP | Subnet | Network Addr. | Host Range | Broadcast |
| --- | --- | --- | --- | --- | --- |
| System 1 | 192.168.1.130 | 192.168.1.128/26 | 192.168.1.128 | 192.168.1.129–190 | 192.168.1.191 |
| System 2 | 192.168.1.190 | 192.168.1.128/26 | 192.168.1.128 | 192.168.1.129–190 | 192.168.1.191 |

Check: 130 lies between 128 and 191, so it falls inside the subnet — valid. 190 also lies between 128 and 191 (it is the last usable host, just before the broadcast 191) — valid.

Conclusion: Both System 1 and System 2 belong to the SAME subnet, 192.168.1.128/26, which provides 2^6 − 2 = 62 usable host addresses.

# Question 3

Organization block 172.16.0.0/16 is split among four departments: HR (/25), Sales (/26), IT (/27), Admin (/28), assigned sequentially.

Solution

| Dept / Mask | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- |
| HR /25 | 172.16.0.0 | 172.16.0.1 – .126 | 172.16.0.127 | 126 |
| Sales /26 | 172.16.0.128 | 172.16.0.129 – .190 | 172.16.0.191 | 62 |
| IT /27 | 172.16.0.192 | 172.16.0.193 – .222 | 172.16.0.223 | 30 |
| Admin /28 | 172.16.0.224 | 172.16.0.225 – .238 | 172.16.0.239 | 14 |

Remaining unused range:

The full block 172.16.0.0/16 spans 172.16.0.0 – 172.16.255.255. After allocating HR, Sales, IT and Admin (up to 172.16.0.239), the unused address space is 172.16.0.240 – 172.16.255.255, available for future departments or expansion.

# Question 4

Company network 172.16.10.0/24 is subnetted using a /27 mask.

Solution

Borrowed bits = 27 − 24 = 3  →  Total subnets = 2^3 = 8.   Host bits = 32 − 27 = 5  →  Usable hosts per subnet = 2^5 − 2 = 30. Block size = 32.

| Subnet # | Network Address | Host Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 172.16.10.0 | 172.16.10.1–30 | 172.16.10.31 |
| 2 | 172.16.10.32 | 172.16.10.33–62 | 172.16.10.63 |
| 3 (contains .78) | 172.16.10.64 | 172.16.10.65–94 | 172.16.10.95 |
| … | … | … | … |

IP 172.16.10.78 falls in the range 172.16.10.64 – 172.16.10.95, so:

- Network Address = 172.16.10.64

- Broadcast Address = 172.16.10.95

Check for 172.16.10.95: this address equals the broadcast address of the same subnet (172.16.10.64/27), so YES — it falls within the same subnet range as 172.16.10.78 (though it is the broadcast address itself, not an assignable host address).

# Question 5

Given IPv6 address: 2001:0db8:0000:0000:0000:ff00:0042:8329

Solution

Compression rules: (1) remove leading zeros in each group, (2) replace the longest run of consecutive all-zero groups with '::' (only once).

| Step | Result |
| --- | --- |
| Original (full form) | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Remove leading zeros | 2001:db8:0:0:0:ff00:42:8329 |
| Compress zero groups | 2001:db8::ff00:42:8329 |
| Expanded back to full form | 2001:0db8:0000:0000:0000:ff00:0042:8329 |

Network prefix at /64:

The first 64 bits (first four groups) form the network prefix: 2001:0db8:0000:0000::/64  (compressed: 2001:db8::/64).

Total possible host addresses:

With a /64 prefix, 128 − 64 = 64 bits remain for host addressing. Number of possible host addresses = 2^64 = 18,446,744,073,709,551,616.

# Summary Table — Key Results

| Q.No | Key Answer |
| --- | --- |
| 1 | Lab A: .0/26 (62 hosts); Lab B: .64/27 (30 hosts); Lab C: .96/28 (14 hosts) |
| 2 | Both systems in subnet 192.168.1.128/26 (62 usable hosts); both IPs valid |
| 3 | HR .0/25, Sales .128/26, IT .192/27, Admin .224/28; unused: 172.16.0.240–172.16.255.255 |
| 4 | 8 subnets of /27, 30 hosts each; .78 → subnet 172.16.10.64/27; .95 = broadcast, same subnet |
| 5 | Compressed: 2001:db8::ff00:42:8329; Prefix: 2001:db8::/64; Hosts: 2^64 |